Model: Fingerprint Lock CJ-6698

Installation Instruction:

Mortise Choice

Hole Sketch

Installation Drawing

Dimension

Technical Specification:

| Item# | Description | Technical Parameters |
| --- | --- | --- |
| 1 | Sensor | Bright background,for totally resolving the issue of remaining fingerprints |
| 2 | Resolution factor | 560 dpi |
| 3 | Sensor resistance to destruction | 4H pencil 20cm constant impact for 1000 times resulting in no destruction |
| 4 | Startup time | ＜1s |
| 5 | Contrast | 1:N |
| 6 | FRR | Less than or equal to 1%(≤1%) |
| 7 | FAR | Less than or equal to 0.0001%(≤0.0001%) |
| 8 | Angle of finger application | ±60° |
| 9 | Fingerprint registration module | One time of registration producing one module |
| 10 | Fingerprint update | Fully automatic update technology |
| 11 | ingerprint template storage | Automatic storage of fingerprint template when batteries are changed |
| 12 | Max figerprint capacity | 150 fingerprints |
| 13 | Light interference | Protection against direct sunshine |
| 14 | Collector | Hard glass,nano-treated surface,extremely resistant to friction |
| 15 | Static power consumption | Less than 30uA |
| 16 | Dynamic power consumption | Working current between 150-300mA |
| 17 | Working voltage | Intemational standard batteries,4*1.5V alkaline batteries,DC6V |
| 18 | Service life of batteries | For opening a door 15,000 times in nomal cases |
| 19 | Power supply | Batteries,plus extemal power supply |
| 20 | Password capacity | 78 |
| 21 | Spare voltage for working voltage | 9V |
| 22 | Waming voltage | 4.8V |
| 23 | Electrostatic resistance | ＞15000V |
| 24 | Working temperature for fingerprint | -20℃～60℃ |
| 25 | Working temperature for keypad | -30℃～70℃ |
| 26 | Working humidity | 10%-97% |
| 27 | Storage temperature | -30℃～70℃ |
| 28 | Password setting | Random combination |
| 29 | password length | 8 digits |
| 30 | Spare power supply for working voltage | 9V |
| 31 | Area of collection | 18.2mm(L)X16mm(W) |