Model: Fingerprint Lock CJ-6686

Installation Instruction:

Mortise Choice

If needing 60mm latch,Please fix the screw to the picture

If needing 70mm latch,Please fix the screw to the picture

Hole Sketch

Installation Drawing Dimension. Technical Specifiation: Item#; Description; Technical Parameters. 1; Sensor; Bright background,for totally resolving the issue of remaining fingerprints. 2; Resolution factor; 560 dpi. 3; Sensor resistance to destruction; 4H pencil 20cm constant impact for 1000 times resulting in no destruction. 4; Startup time; ＜1s. 5; Contrast; 1:N. 6; FRR; Less than or equal to 1%(≤1%). 7; FAR; Less than or equal to 0.0001%(≤0.0001%). 8; Angle of finger application; ±60°. 9; Fingerprint registration module; One time of registration producing one module. 10; Fingerprint update; Fully automatic update technology. 11; ingerprint template storage; Automatic storage of fingerprint template when batteries are changed. 12; Max figerprint capacity; 150 fingerprints. 13; Light interference; Protection against direct sunshine. 14; Collector; Hard glass,nano-treated surface,extremely resistant to friction. 15; Static power consumption; Less than 30uA. 16; Dynamic power consumption; Working current between 150-300mA. 17; Working voltage; Intemational standard batteries,4*1.5V alkaline batteries,DC6V. 18; Service life of batteries; For opening a door 15,000 times in nomal cases. 19; Power supply; Batteries,plus extemal power supply. 20; Password capacity; 78. 21; Spare voltage for working voltage; 9V. 22; Waming voltage; 4.8V. 23; Electrostatic resistance; ＞15000V. 24; Working temperature for fingerprint; -20℃～60℃. 25; Working temperature for keypad; -30℃～70℃. 26; Working humidity; 10%-97%. 27; Storage temperature; -30℃～70℃. 28; Password setting; Random combination. 29; password length; 8 digits. 30; Spare power supply for working voltage; 9V. 31; Area of collection; 18.2mm(L)X16mm(W).